2048-ppt FFT Design Document

Mobile Satellite Services Corporation

18221A Flower Hill Way

Gaithersburg, MD 20879

U.S.A.

Telephone:		240.631.1111

Facsimile:		240.631.1676
Internet E-mail:	info@mobile-sat.com

MOBILE SATELLITE SERVICES

Corporation

Proprietary and Confidentiality Notice
This document is proprietary to Mobile Satellite Services Corporation. Use or disclosure of the information contained therein is not permitted without the prior authorization of Mobile Satellite Services Corporation. MSS CONFIDENTIAL.

- Revision History

| Created | 6/00 Michael Yoha | 1st release. |
| --- | --- | --- |
| Revision | Date / Author | Change summary |
| 2 | AZ. 2/18/01 | Added figures + Matlab simulation code + VHDL simulation results |
| 3 | PS 11/28/05 | Changes based on COM-1221_010 by AZ |

1	Scope	3

2	FFT	4

2.1	Background	4

2.2	Radix 2 Butterfly	4

2.3	Radix 4 Butterfly	4

3	FFT Stages	6

4	Matlab Simulation	8

5	VHDL Simulation	13

5.1	Test Input	13

5.2	FFT Control	13

5.3	FFT Radix2_4	16

5.4	FFT Coefficients	18

5.5	Test Results	20

# Scope

This document is intended for those who wish to understand the design and implementation of the FFT module “fft_control2.vhd”

The salient features of the module are:

- 2048-ppt complex FFT using internal FPGA memory and multipliers

- 12-bit input and output data precision

- User specified index associated with each complex input and output data samples

- Computation speed is one FFT frame every 70 us

- 6 stages (1 Radix-2 at 1st stage followed by 5 Radix-4)

- Choice of forward or Inverse FFT

- Overflow indication at every stage

- User adjustable gain for next FFT computation, based on the overflow information of current FFT

# FFT

## Background

The Fast Fourier Transform (FFT) is another method to calculate the Discrete Fourier Transform (DFT).  FFT is faster, more efficient and based on the complex DFT. The typical equation used to calculate the FFT is:

N-1               nk

X(k) =       X(n) WN		0 k  N-1

n=0

nk       -j(2π/N)kn

WN = e

Where X(k) is the output response at time k and  X(n) is the complex signal , WN    is the constant valued coefficients. A single output response requires the addition of many k terms, which can be referred to as the sum of products. The difference between FFT and IFFT is the sign change for WN.

nk       j(2π/N)kn

WN = e

## Radix 2 Butterfly

The FFT is a block process that can be computed by using the Radix 2 butterfly.

Am = Xm + Xn  =  XRm + XRn + j( XIm + XIn )

An = (Xm –Xn) Wk  ; W k  = e –j2k/N  = cosk –j sink

An= (XRm – XRn)cosk + (XIm – XIn )sink + j[(XRm + XRn )(-sink) + (XIm –XIn)cosk]

## Radix 4 Butterfly

A similar process to the Radix 2 butterfly is called the Radix 4 butterfly. Four inputs are used to compute four outputs.

Where:

x(n) 		= xa + j ya

x(n+N/4) 	= xb + j yb

x(n+N/2) 	= xc + j yc

x(n+3N/4) 	= xd + j yd

x(4r)		= xa’ + j ya’

x(4r+1)	= xb’ + j yb’

x(4r+2)	= xc’ + j yc’

x(4r+3)	= xd’ + j yd’

xa’ = 	xa+xb+xc+xd

ya’ = 	ya+yb+yc+yd

xb’ =	(xa+yb-xc-yd)Cb – (ya-xb-yc+xd)(-Sb)

yb’ =	(ya-xb-yc+xd)Cb + (xa+yb-xc-yd)(-Sb)

xc’ = 	(xa-xb+xc-xd)Cc – (ya-yb+yc-yd)(-Sc)

yc’ = 	(ya-yb+yc-yd)Cc + (xa-xb+xc-xd)(-Sc)

xd’ =	(xa-yb-xc+yd)Cd – (ya+xb-yc-xd)(-Sd)

yd’ =	(ya+xb-yc-xd)Cd + (xa-yb-xc+yd)(-Sd)

and the coefficients are computed as

W n  = e –j2n/N  = Cb +j (-Sb)

W 2n  = e –j22n/N  = Cc +j (-Sc)

W 3n  = e –j23n/N  = Cd +j (-Sd)

# FFT Stages

VHDL implementation of the 2048 ppt FFT uses 5 radix-4 computations and 1 Radix-2 computation as below:

Stage 1 - Radix 2, 1 part, 2084 ppt FFT

Stage 2 - Radix 4, 2 parts, 1024 ppt FFT

Stage 3 - Radix 4, 4 parts 256 ppt FFT

Stage 4 - Radix 4, 16 parts, 64 ppt FFT

Stage 5 - Radix 4, 64 parts, 16 ppt FFT

Stage 6 - Radix 4, 256 parts, 4 ppt FFT

The maximum FFT computation speed for every FFT stage is 1050 CLKs( 1024 + margin) which accounts to (1050*6) = 6300 CLKs for all the computation stages combined.

The clock speed for this module is currently fixed at 90 MHz, hence the total time required for FFT computation is about 70 us.

Two Radix-2 butterflies are computed every two CLKs and one Radix-4 butterfly is calculated every two CLKs

The resource utilization for this FFT module on a Xilinx Spartan-3 1500 device is:

Number of Slices:                        1246  out of  13312     9%

Number of Slice Flip Flops:        1440  out of  26624     5%

Number of 4 input LUTs:            1918  out of  26624     7%

Number of bonded IOBs:             95  out of    487    19%

Number of BRAMs:                     8  out of     32    25%

Number of MULT18X18s:           6  out of     32    18%

Number of GCLKs:                      1  out of      8    12%

Maximum Frequency: 	       80.166MHz

Figure 1: Butterfly Implementation

# Matlab Simulation

% Copyright Mobile Satellite Services Corporation 2001

% file: FFTSIM.M

% Author: Alain Zarembowitch / MSS

% Last edited: 2-18-01

% Revision: 0.001

% FFT simulation

%--------------------------------------------------

% place reference signal here

f = 1/200;

X = sin(2*pi*f*(1:2048));

% reference

Yref = fft(X, 2048);

% 1st stage = radix 2

for k = 1:1024

Wn = cos(-2*pi*(k-1)/2048) + j*sin(-2*pi*(k-1)/2048);

X1(k) = X(k) + X(k+1024);

X1(k+1024) = (X(k) - X(k+1024))*Wn;

end

% 2nd stage = radix 4, 2 parts, 1024 ppt FFT

for l = 0:1

for k = 1+l*1024:256+l*1024

Cb = cos(2*pi*(k-1)/1024);

Sb = sin(2*pi*(k-1)/1024);

Cc = cos(2*pi*2*(k-1)/1024);

Sc = sin(2*pi*2*(k-1)/1024);

Cd = cos(2*pi*3*(k-1)/1024);

Sd = sin(2*pi*3*(k-1)/1024);

xa = real(X1(k));

ya = imag(X1(k));

xb = real(X1(k+256));

yb = imag(X1(k+256));

xc = real(X1(k+512));

yc = imag(X1(k+512));

xd = real(X1(k+768));

yd = imag(X1(k+768));

x2a = xa + xb + xc + xd;

y2a = ya + yb + yc + yd;

x2b = (xa+yb-xc -yd)*Cb - (ya-xb-yc+xd)*(-Sb);

y2b = (ya-xb-yc+xd)*Cb + (xa+yb-xc-yd)*(-Sb);

x2c = (xa-xb+xc-xd)*Cc - (ya-yb+yc-yd)*(-Sc);

y2c = (ya-yb+yc-yd)*Cc + (xa-xb+xc-xd)*(-Sc);

x2d = (xa-yb-xc+yd)*Cd - (ya+xb-yc-xd)*(-Sd);

y2d = (ya+xb-yc-xd)*Cd + (xa-yb-xc+yd)*(-Sd);

X2(k) = x2a + j*y2a;

X2(k+256) = x2b + j*y2b;

X2(k+512) = x2c + j*y2c;

X2(k+768) = x2d + j*y2d;

end

end

% 3rd stage = radix 4, 4 parts, 256 ppt FFT

for l = 0:7

for k = 1+l*256:64+l*256

Cb = cos(2*pi*(k-1)/256);

Sb = sin(2*pi*(k-1)/256);

Cc = cos(2*pi*2*(k-1)/256);

Sc = sin(2*pi*2*(k-1)/256);

Cd = cos(2*pi*3*(k-1)/256);

Sd = sin(2*pi*3*(k-1)/256);

xa = real(X2(k));

ya = imag(X2(k));

xb = real(X2(k+64));

yb = imag(X2(k+64));

xc = real(X2(k+128));

yc = imag(X2(k+128));

xd = real(X2(k+192));

yd = imag(X2(k+192));

x2a = xa + xb + xc + xd;

y2a = ya + yb + yc + yd;

x2b = (xa+yb-xc -yd)*Cb - (ya-xb-yc+xd)*(-Sb);

y2b = (ya-xb-yc+xd)*Cb + (xa+yb-xc-yd)*(-Sb);

x2c = (xa-xb+xc-xd)*Cc - (ya-yb+yc-yd)*(-Sc);

y2c = (ya-yb+yc-yd)*Cc + (xa-xb+xc-xd)*(-Sc);

x2d = (xa-yb-xc+yd)*Cd - (ya+xb-yc-xd)*(-Sd);

y2d = (ya+xb-yc-xd)*Cd + (xa-yb-xc+yd)*(-Sd);

X3(k) = x2a + j*y2a;

X3(k+64) = x2b + j*y2b;

X3(k+128) = x2c + j*y2c;

X3(k+192) = x2d + j*y2d;

end

end

% 4rd stage = radix 4, 16 parts, 64 ppt FFT

for l = 0:31

for k = 1+l*64:16+l*64

Cb = cos(2*pi*(k-1)/64);

Sb = sin(2*pi*(k-1)/64);

Cc = cos(2*pi*2*(k-1)/64);

Sc = sin(2*pi*2*(k-1)/64);

Cd = cos(2*pi*3*(k-1)/64);

Sd = sin(2*pi*3*(k-1)/64);

xa = real(X3(k));

ya = imag(X3(k));

xb = real(X3(k+16));

yb = imag(X3(k+16));

xc = real(X3(k+32));

yc = imag(X3(k+32));

xd = real(X3(k+48));

yd = imag(X3(k+48));

x2a = xa + xb + xc + xd;

y2a = ya + yb + yc + yd;

x2b = (xa+yb-xc -yd)*Cb - (ya-xb-yc+xd)*(-Sb);

y2b = (ya-xb-yc+xd)*Cb + (xa+yb-xc-yd)*(-Sb);

x2c = (xa-xb+xc-xd)*Cc - (ya-yb+yc-yd)*(-Sc);

y2c = (ya-yb+yc-yd)*Cc + (xa-xb+xc-xd)*(-Sc);

x2d = (xa-yb-xc+yd)*Cd - (ya+xb-yc-xd)*(-Sd);

y2d = (ya+xb-yc-xd)*Cd + (xa-yb-xc+yd)*(-Sd);

X4(k) = x2a + j*y2a;

X4(k+16) = x2b + j*y2b;

X4(k+32) = x2c + j*y2c;

X4(k+48) = x2d + j*y2d;

end

end

% 5th stage = radix 4, 64 parts, 16 ppt FFT

for l = 0:127

for k = 1+l*16:4+l*16

Cb = cos(2*pi*(k-1)/16);

Sb = sin(2*pi*(k-1)/16);

Cc = cos(2*pi*2*(k-1)/16);

Sc = sin(2*pi*2*(k-1)/16);

Cd = cos(2*pi*3*(k-1)/16);

Sd = sin(2*pi*3*(k-1)/16);

xa = real(X4(k));

ya = imag(X4(k));

xb = real(X4(k+4));

yb = imag(X4(k+4));

xc = real(X4(k+8));

yc = imag(X4(k+8));

xd = real(X4(k+12));

yd = imag(X4(k+12));

x2a = xa + xb + xc + xd;

y2a = ya + yb + yc + yd;

x2b = (xa+yb-xc -yd)*Cb - (ya-xb-yc+xd)*(-Sb);

y2b = (ya-xb-yc+xd)*Cb + (xa+yb-xc-yd)*(-Sb);

x2c = (xa-xb+xc-xd)*Cc - (ya-yb+yc-yd)*(-Sc);

y2c = (ya-yb+yc-yd)*Cc + (xa-xb+xc-xd)*(-Sc);

x2d = (xa-yb-xc+yd)*Cd - (ya+xb-yc-xd)*(-Sd);

y2d = (ya+xb-yc-xd)*Cd + (xa-yb-xc+yd)*(-Sd);

X5(k) = x2a + j*y2a;

X5(k+4) = x2b + j*y2b;

X5(k+8) = x2c + j*y2c;

X5(k+12) = x2d + j*y2d;

end

end

% 6th stage = radix 4, 256 parts, 4 ppt FFT

for l = 0:511

for k = 1+l*4:1+l*4

Cb = cos(2*pi*(k-1)/4);

Sb = sin(2*pi*(k-1)/4);

Cc = cos(2*pi*2*(k-1)/4);

Sc = sin(2*pi*2*(k-1)/4);

Cd = cos(2*pi*3*(k-1)/4);

Sd = sin(2*pi*3*(k-1)/4);

xa = real(X5(k));

ya = imag(X5(k));

xb = real(X5(k+1));

yb = imag(X5(k+1));

xc = real(X5(k+2));

yc = imag(X5(k+2));

xd = real(X5(k+3));

yd = imag(X5(k+3));

x2a = xa + xb + xc + xd;

y2a = ya + yb + yc + yd;

x2b = (xa+yb-xc -yd)*Cb - (ya-xb-yc+xd)*(-Sb);

y2b = (ya-xb-yc+xd)*Cb + (xa+yb-xc-yd)*(-Sb);

x2c = (xa-xb+xc-xd)*Cc - (ya-yb+yc-yd)*(-Sc);

y2c = (ya-yb+yc-yd)*Cc + (xa-xb+xc-xd)*(-Sc);

x2d = (xa-yb-xc+yd)*Cd - (ya+xb-yc-xd)*(-Sd);

y2d = (ya+xb-yc-xd)*Cd + (xa-yb-xc+yd)*(-Sd);

X6(k) = x2a + j*y2a;

X6(k+1) = x2b + j*y2b;

X6(k+2) = x2c + j*y2c;

X6(k+3) = x2d + j*y2d;

end

end

% reshuffle indices

for k = 1:2048

input_index = dec2bin(k-1,11);

output_index(1) = input_index(10);

output_index(2) = input_index(11);

output_index(3) = input_index(8);

output_index(4) = input_index(9);

output_index(5) = input_index(6);

output_index(6) = input_index(7);

output_index(7) = input_index(4);

output_index(8) = input_index(5);

output_index(9) = input_index(2);

output_index(10) = input_index(3);

output_index(11) = input_index(1);

k2 = bin2dec(output_index) + 1;

X7(k2) = X6(k);

end

% compare Matlab FFT with this radix2/radix4 FFT

plot(abs(X7));

hold;

plot(abs(Yref),'r');

Figure 2: Matlab simulation: identical results for Matlab fft function (red) and proposed mixed-radix algorithm (blue, superposed).

# VHDL Simulation

## Test Input

An impulse signal is provided in the second bin of DATA_I_IN signal. Inverse FFT is calculated. The expected result is a sine wave on I and Q outputs.

Figure 3: Input signals

## FFT Control

Module : FFT_CONTROL2.vhd

In order to understand the working of this module, it is important to note the following:

- FFT load/unload and computation is done in 7 stages.

Stage 0 : Radix-2 computation

Stages 1-5 : Radix-4 computation

Stage 6 : Data loading/unloading

- The frequency domain representation of the time-domain signal is such that 0 up to 1023 represent increasing positive frequencies and 2047 down to 1024 represent decreasing negative frequencies

- We need to store: 2 (odd/even addresses) * 2 (I/Q complex components) * 2048 (FFT size) * 12 (bit precision) = 98304 bits = 6 RAMBs.

- Buffers are used to store intermediate results while computing an FFT in place, and to store a complete frame of input samples prior to starting the next FFT.

- 3*8-bit RAMB are used to store 2*12-bit precision samples.

- Alternate RAMB’s are selected at every stage. Reason: the 2048-ppt FFT has an even number of processing stages(6). Therefore, we could not load and unload data simultaneously if we did not switch at every frame.

- RAMB_RW_SELECT selects the ramb to read and write. When ‘1’ write to RAMBs1-3, read from RAMBs4-6.

- Read/write up to two addresses simultaneously using the Dual-port RAM Generally, ADDRA is even, ADDRB is odd, except during input/outputs where only ADDRA is used and can take odd and even values.

Figure 4: FFT_Control2 signals

- K and K+1 inputs are read at the same time using the -A and -B sides of the dual port RAM

- RAMB input data is obtained by combining 2*12-bit precision complex samples into 3*8 bit words

- RAMB output data is reconstructed into the original 2*12-bit precision complex samples from 3*8 bit words

- RAMB inputs and outputs are selected depending on the FFT stage.

- FFT start pulse, which is one CLK wide, is used to indicate the start of FFT computation based on the input frame already loaded up. Typically, the user will send this pulse once the previous FFT output has been read and the new FFT input data has been uploaded.

- During FFT computation stages, data is written/read simultaneously at odd and even addresses

- Duration for each stage: 1024 CLKs + processing delay = 1050 CLKs PHASE_COUNTER is used to keep track of the these clks.

- Based on the PHASE_COUNTER it takes 4 CLKs(PHASE_COUNTER_D4) for butterfly computation to start and 11 CLKs(PHASE_COUNTER_D11) to start the write cycle

- At every stage of computation for every 2 CLKs, 4 complex samples are read from the RAMB. For Radix-2: 4 complex samples are read as input to two radix-2 butterflies. For Radix-4: 4 complex samples are read as input to one radix-4 butterfly. These complex samples are pushed into a stack.

- Stage 0 : Radix-2

  - Address sequence (0,1 (same clock), (1024, 1025), (2,3),(1026, 1027), etc.

- Stage 1 : Radix-4

  - Address sequence (0, 256, 512, 768), (1, 257, 513, 769), etc.

  - 2 parts

- Stage 2 : Radix-4

  - Address sequence (0, 64, 128, 192), (1, 65, 129, 193), etc.

  - 4 parts

- Stage 3 : Radix-4

  - Address sequence (0, 16, 32, 48), (1, 17, 33, 49), etc.

  - 16 parts

- Stage 4 : Radix-4

  - Address sequence (0, 4, 8, 12), (1, 5, 9, 13), etc.

  - 64 parts

- Stage 5 : Radix-4

- Address sequence (0, 1, 2, 3), (4, 5, 6, 7), etc.

- 256 parts

- Stage 6 : Data Load/Unload

  - Data is written in a user specified write sequence(FFT_INDEX_IN). Data is written only on A side of the dual-port RAM

  - Input data samples and indices are read at the rising edge of CLK when SAMPLE_CLK_IN = '1'. Note that this component does not count the number of input samples. The user can thus write, overwrite or underwrite the input elastic buffer.

  - IO_ENABLED tells the user if FFT input samples can be read and if the FFT output data can be downloaded.

  - Input FFT index ranges from 0 to 2047 (0 up to 1023 represent increasing positive frequencies, 2047 down to 1024 represent decreasing negative frequencies).

  - if user requests data then data is read in a user-specify read sequence(FFT_INDEX_OUT)

  - Output FFT index ranges from 0 to 2047. Indices are reshuffled since the FFT computations are done in place.

- OVERFLOW is indicated for each FFT stage (bit 0 for stage 0, bit 1 for stage 1, etc.) OVERFLOW contents are valid when IOENABLED = '1' (contents refer to previous FFT). It is cleared at FFT_START.

- FFT_SCALE which is read at the start of computation, allows the user to adjust the gain at each FFT stage, based on the overflow information supplied within the OVERFLOW output. This controls the gain of the next FFT, starting at FFT_START, whereas the OVERFLOW information relates to the previous FFT.

- Output data is read at the rising edge of CLK when SAMPLE_CLK_OUT = '1'.

- SAMPLE_CLK_OUT_REQ is the pulse to request FFT output data. FFT_INDEX_OUT must be valid when SAMPLE_CLK_OUT_REQ = '1'.  Latency between SAMPLE_CLK_OUT_REQ and SAMPLE_CLK_OUT is 3 CLKs.

## FFT Radix2_4

Computes new set of butterflies every 2 CLKs

X1 + jY1 inputs

X2 + jY2 outputs

Coefficients  Wk = e(j2pik/N) = Cosine + j Sine

The coefficients are read at SAMPLE_CLK_IN and stored locally.

OUTPUT_SHIFT is used to shift output 0,1,2 or 3 bits to the right. When using 0, no overflow can occur. Check OVERFLOW bit to select the OUTPUT_SHIFT.

Radix-2

- Two butterflies calculated every 2 counter values

- 1st butterfly

  - inputs are (xa + jya) and (xb + jyb)

  - coefficients are CB/SB

  - outputs are (x2a + jy2a), (x2b + jy2b)

x2a = xa + xb

y2a = ya + yb

x2b = (xa-xb)*Cb + (ya-yb)*Sb

y2b = (ya-yb)*Cb - (xa-xb)*Sb

- 2nd butterfly

  - inputs are (xc + jyc) and (xd + jyd)

  - coefficients are CD/SD

  - outputs are (x2c + jy2c), (x2d + jy2d)

x2c = xc + xd

y2c = yc + yd

x2d = (xc-xd)*Cd + (yc-yd)*Sd

y2d = (yc-yd)*Cd - (xc-xd)*Sd

- Key assumption : input samples X,Y must be frozen for 2 clks starting at SAMPLE_CLK_IN.

Figure 5:Radix-2 computation results

Radix-4

- One butterfly calculated every 2 counter values

- Inputs are (xa + jya), (xb + jyb), (xc + jyc) and (xd + jyd)

- Coefficients are Cb/Sb, Cc/Sc and Cd/Sd

- 1st phase

  - ((xa-xc)+(yb-yd))*Cb or -Sb

  - ((ya-yc)+(xd-xb))*Cb or -Sb

  - ((xa-xc)-(yb-yd))*Cd or -Sd

  - ((ya-yc)-(xd-xb))*Cd or –Sd

- 2nd phase:

  - ((xa+xc)-(xb+xd))*Cc or -Sc

  - ((ya+yc)-(yb+yd))*Cc or –Sc

  - ((xa+xc)+(xb+xd))

  - ((ya+yc)+(yb+yd))

- Outputs are (x2a + jy2a), (x2b + jy2b)

x2a = xa + xb + xc + xd

y2a = ya + yb + yc + yd

x2b = (xa + yb - xc -yd)*Cb - (ya - xb - yc + xd)*(-Sb)

y2b = (ya - xb + xc -xd)*Cb + (xa + xb - xc - yd)*(-Sb)

Figure 6: Radix-4 computation results

## FFT Coefficients

Module: FFTCOEFF.VHD

FFT_STAGE = 0 (Radix 2)

Cb = cos(-2*pi*k/2048), Sb = sin(-2*pi*k/2048)

Cc = cos(-2*pi*(k+1)/2048), Sc = sin(-2*pi*(k+1)/2048)

Cd = Cc, Sd = Sc

Input K ranges from 0 to 1022 by steps of 2 (thus k+1 goes to 1023)

FFT_STAGE = 1 (Radix 4)

Cb = cos(2*pi*k/1024), Sb = sin(2*pi*k/1024)

Cc = cos(2*pi*2*k/1024), Sc = sin(2*pi*2*k/1024)

Cc = cos(2*pi*3*k/1024), Sc = sin(2*pi*3*k/1024)

Input K ranges from 0 to 255 in steps of 1

FFT_STAGE = 2 (Radix 4)

Cb = cos(2*pi*k/256), Sb = sin(2*pi*k/256)

Cc = cos(2*pi*2*k/256), Sc = sin(2*pi*2*k/256)

Cc = cos(2*pi*3*k/256), Sc = sin(2*pi*3*k/256)

Input K ranges from 0 to 63 in steps of 1

FFT_STAGE = 3 (Radix 4)

Cb = cos(2*pi*k/64),	Sb = sin(2*pi*k/64)

Cc = cos(2*pi*2*k/64), Sc = sin(2*pi*2*k/64)

Cc = cos(2*pi*3*k/64), Sc = sin(2*pi*3*k/64)

Input K ranges from 0 to 15 in steps of 1

FFT_STAGE = 4 (Radix 4), this module computes

Cb = cos(2*pi*k/16),	Sb = sin(2*pi*k/16)

Cc = cos(2*pi*2*k/16), Sc = sin(2*pi*2*k/16)

Cc = cos(2*pi*3*k/16), Sc = sin(2*pi*3*k/16)

Input K ranges from 0 to 3 in steps of 1

FFT_STAGE = 5 (Radix 4)

Cb = 1, Sb = 0

Cc = 1, Sc = 0

Cc = 1, Sc = 0

K goes from 0-1023 by steps of 2(Radix-2) or 1(Radix-4) one computation every 2 CLKs

Figure 7: Coefficients

## Test Results

Figure 8: Output signals

Figure 9: Matlab representation of the output signals